# A Complete Learning Guide to the Four Bit Counter UVM Testbench

From basic Verilog to reading, explaining, and eventually understanding every file in this project

Reader starting point: I know the basics of Verilog. I am new to SystemVerilog and UVM. I want to learn them through this real counter testbench, understand the purpose and behavior of every component, and use this guide as reliable source material for a podcast or tutoring discussion.

The central idea: the sequence asks for a reset value, the driver applies it to a real wire, the counter reacts at a rising clock edge, the monitor records what actually happened, and the scoreboard independently checks the result. The remaining UVM code creates, connects, and manages those jobs.

## How to use this guide

1. Read Parts 1 through 3 to connect the project to the Verilog you already know.

2. Trace the single clock cycle in Part 4 before trying to memorize class names.

3. Use the file walkthrough in Part 5 while opening each source file beside this document.

4. Use the podcast plan near the end as an outline, not as a substitute for checking the code.

# Part 1 The Verilog behavior we must verify

## The design under test

The DUT is rtl/counter.v. It has clk, active-high reset, and a four-bit output count. A copy of the original counter was placed inside this project without changing its logic.

always @(posedge clk) begin
    if (reset)
        count <= 4'b0000;
    else
        count <= count + 1'b1;
end

The counter changes only at a positive clock edge. Reset is synchronous: raising reset halfway through a clock period does not immediately clear count. Reset must be high when a positive edge occurs. The nonblocking assignment operator <= schedules the new count for the nonblocking-assignment region, after the active-region evaluation of that edge.

| Reset at rising edge | Previous count | Expected count after edge |
| --- | --- | --- |
| 1 | Any value | 0 |
| 0 | 0 | 1 |
| 0 | 14 | 15 |
| 0 | 15 | 0 |

The last row is rollover. A four-bit number holds 0 through 15; adding one to 15 discards the carry and produces 0. This is modulo-16 arithmetic.

## What a meaningful test must do

Provide reset values before rising edges, including an initial reset and a reset after counting has started.

Observe the output after each rising-edge update, not before the nonblocking assignment settles.

Maintain an expected count that is independent of the DUT output and compare it with the observed output.

Reach count 15, verify the next count is 0, and confirm counting resumes after reset.

# Part 2 A small SystemVerilog bridge

You can read this project without mastering all of SystemVerilog. The table below translates the features that appear here into familiar ideas.

| Construct | How to read it in this project |
| --- | --- |
| logic | A signal type used for reset and count in the interface. |
| bit | A two-state 0/1 variable. The item reset and observed fields use bit. |
| interface | A named bundle of related wires: clk, reset, and count. |
| class | A definition for a testbench object, similar to a software blueprint. |
| extends | Inherits behavior from a UVM base class. |
| function / task | A function completes without waiting for simulation time; a task may wait for edges. |
| new | Constructor that initializes an object or component. |
| #(counter_seq_item) | A parameterized UVM class specialized to carry counter items. |
| package / import | A namespace for the testbench classes, and a way to make names visible. |
| virtual interface | A class variable that refers to an existing interface instance. |
| covergroup | A SystemVerilog mechanism for recording which situations occurred. |

## The names cif and vif

tb_top.sv creates one real interface instance called cif and connects its pins to the DUT. The driver and monitor are classes defined elsewhere, so they receive references to that existing instance. Each class stores its reference in a variable named vif. Thus vif.reset in the driver is the same physical reset signal as cif.reset in tb_top. No second wire is created.

counter_if cif(clk);                     // real instance in tb_top
virtual counter_if vif;                // handle declared in a class
vif.reset = req.reset;                 // change the DUT input

The top passes the reference using uvm_config_db::set. The driver and monitor each retrieve it with uvm_config_db::get. A failed get produces UVM_FATAL rather than letting an unconnected component run silently.

## Classes and objects

counter_seq_item is a class. A particular item object can hold reset = 1; another item object can hold reset = 0. The sequence creates a fresh object for each requested cycle. The UVM driver, monitor, scoreboard, agent, environment, and test are also classes, but they are long-lived UVM components with a place in the component hierarchy.

# Part 3 The architecture and the two information flows

REQUEST PATH
counter_test -> counter_sequence -> counter_sequencer
             -> counter_driver -> counter_if -> counter DUT

OBSERVATION PATH
counter DUT -> counter_if -> counter_monitor
            -> analysis port -> analysis FIFO -> counter_scoreboard

The request path carries an instruction: what reset value should be applied? The observation path carries evidence: what values appeared on the real pins? The scoreboard must use the evidence, not merely trust the request.

| Block | Responsibility | Important source file |
| --- | --- | --- |
| Top module | Create clock, interface, DUT, configuration entries, and start UVM. | tb_top.sv |
| Test | Create the environment, start the sequence, keep the run alive. | counter_test.svh |
| Sequence item | Hold one requested reset value. | counter_seq_item.svh |
| Sequence | Choose the directed reset/count pattern. | counter_sequence.svh |
| Sequencer | Coordinate delivery of items to the driver. | counter_sequencer.svh |
| Driver | Convert each item into real reset signal activity. | counter_driver.svh |
| Monitor | Read real reset/count pins after each rising edge. | counter_monitor.svh |
| Observation | Hold one actual reset/count sample. | counter_observation.svh |
| Analysis FIFO | Queue observations until the scoreboard consumes them. | counter_scoreboard.svh |
| Scoreboard | Predict count independently and compare with actual count. | counter_scoreboard.svh |
| Agent / environment | Create and connect the components. | counter_agent.svh / counter_env.svh |

A useful analogy is an experiment. The sequence writes an instruction card. The driver operates the switch. The DUT is the device being tested. The monitor writes down the reading. The scoreboard checks the reading against its own calculation. The sequencer and FIFO keep the two streams orderly.

# Part 4 Trace one complete clock cycle

Assume the scoreboard currently expects count = 3. The sequence wants one normal count cycle, so it requests reset = 0.

1. counter_sequence creates a counter_seq_item and assigns item.reset = 0. This item describes the request; it is not a wire.

2. start_item and finish_item use the sequencer handshake. The sequencer makes the item available to the driver.

3. counter_driver receives it with seq_item_port.get_next_item(req). req.reset now equals 0.

4. At the next falling clock edge, the driver executes vif.reset = req.reset. The real DUT reset input becomes 0 and remains stable until the next rising edge.

5. At the rising edge, the DUT evaluates its always block. It schedules count <= count + 1. Given the prior value 3, the new output becomes 4 in the nonblocking-assignment region.

6. The driver calls item_done after that rising edge, allowing the sequence to move to another request.

7. The monitor waits 1 ps after the rising edge, then reads the actual interface pins. It records reset = 0 and count = 4 in a counter_observation.

8. The monitor calls observed.write(sample). Its analysis connection inserts the observation into the scoreboard's uvm_tlm_analysis_fifo.

9. The scoreboard obtains the observation with observations.get(sample), computes expected = 3 + 1 = 4, and compares actual 4 with expected 4. This cycle passes.

If the monitor had observed count = 5, the scoreboard would issue UVM_FATAL with both values. The expected value must never be calculated from the observed count, because that would let an incorrect output become the next supposed answer.

## Why the falling edge and the 1 ps wait both matter

The falling-edge drive puts reset on the wire before the DUT samples it at the rising edge. The 1 ps monitor delay addresses a different issue: the DUT uses a nonblocking assignment, so the new output appears after the rising-edge evaluation. The monitor samples once the new value has settled. These are deliberate timing choices in this testbench.

## The very first edge

tb_top initializes cif.reset to 1 at time zero. The clock's first rising edge is at 5 ns, so the DUT is reset before the driver applies its first sequence item at the following falling edge. The monitor and scoreboard check this initial edge. The 30 requested sequence items then account for the other 30 checked edges. The simulation ends at 310 ns after 31 checks.

# Part 5 Read every source file with a purpose

## rtl/counter.v The DUT

This is the hardware under test, not a UVM component. Its entire behavior is the positive-edge always block shown in Part 1. Focus on synchronous reset, nonblocking assignment, and four-bit wraparound.

## counter_if.sv The wire bundle

interface counter_if(input logic clk);
  logic reset;
  logic [3:0] count;
endinterface

An interface groups the pins that the class-based testbench needs to drive or observe. clk enters through the interface port; reset and count are members. The top module connects the interface to the DUT.

## tb_top.sv The bridge between modules and classes

counter_if cif(clk);
counter dut(.clk(clk), .reset(cif.reset), .count(cif.count));
uvm_config_db#(virtual counter_if)::set(
    null, "uvm_test_top.env.agent.driver", "vif", cif);
run_test("counter_test");

There is also a matching set for the monitor. The string path identifies the intended UVM component. run_test creates the named test through UVM. The top module is responsible for the clock and real hardware connections, while classes handle test behavior.

## counter_pkg.sv The class collection

The package imports uvm_pkg, includes uvm_macros.svh, and includes the project class files in dependency order. It allows tb_top.sv to import counter_pkg. The include files use the .svh extension because they are included into the package, rather than compiled as independent top-level modules.

## counter_seq_item.svh The request card

class counter_seq_item extends uvm_sequence_item;
  rand bit reset;
  `uvm_object_utils_begin(counter_seq_item)
    `uvm_field_int(reset, UVM_ALL_ON)
  `uvm_object_utils_end
endclass

The item represents exactly one requested reset value. rand means the field could be randomized, but this directed sequence assigns it explicitly. The UVM macros register the type and field for standard library services. The constructor calls super.new(name), initializing the parent class.

## counter_sequence.svh The test pattern

item = counter_seq_item::type_id::create("item");
start_item(item);
item.reset = reset_value;
finish_item(item);

body() calls send_cycle with a deterministic pattern: initial reset, five count requests, a reset during operation, 18 count requests reaching rollover, two more reset requests, and three resumed count requests. This is 30 items total. It uses the normal sequence and driver handshake rather than directly driving pins.

## counter_sequencer.svh The item coordinator

This file deliberately contains little custom code. It extends uvm_sequencer parameterized by counter_seq_item, registers the component, and has a constructor. The sequencer coordinates item delivery. It does not invent stimulus or touch DUT signals.

## counter_driver.svh The instruction to wire converter

seq_item_port.get_next_item(req);
@(negedge vif.clk);
vif.reset = req.reset;
@(posedge vif.clk);
seq_item_port.item_done();

get_next_item blocks until an item is available. req.reset is the request value; vif.reset is the real input. The driver changes reset on the falling edge, waits through the relevant rising edge, and acknowledges the item. Its build_phase retrieves vif and stops fatally if it is missing.

## counter_observation.svh The actual reading

This separate class holds reset and count as observed by the monitor. Separating it from the request item makes it harder to confuse desired stimulus with DUT evidence. The monitor creates a new observation object for each sampled rising edge.

## counter_monitor.svh The passive witness

@(posedge vif.clk);
#1ps;
sample.reset = vif.reset;
sample.count = vif.count;
observed.write(sample);

The monitor never drives reset or count. It samples the interface after the DUT update, publishes an observation through a uvm_analysis_port, and samples its covergroup using the same observed data. Its own config_db get is checked in build_phase.

## counter_scoreboard.svh The independent reference model

observations.get(sample);
if (sample.reset) expected = 0;
else              expected = expected + 4'd1;
if (sample.count !== expected)
  `uvm_fatal("COUNT_MISMATCH", ...);

The scoreboard owns a uvm_tlm_analysis_fifo. Its run_phase repeatedly removes observations, predicts a four-bit expected value, and compares it with the actual count. It waits for an initial reset before using a predicted state. It also counts checks, resets, increments, rollover, reset during operation, resumed counting, and seen output values.

The check_phase verifies that the intended scenarios occurred and that the FIFO is empty. A mismatch or incomplete run reports UVM_FATAL. The report_phase prints the final summary. The midrun_resets counter counts reset edges after counting began, not distinct reset pulses.

## counter_agent.svh The interface team

The agent creates one sequencer, one driver, and one monitor in build_phase. In connect_phase, it connects driver.seq_item_port to sequencer.seq_item_export. The monitor remains passive. The agent does not contain the scoreboard in this project.

## counter_env.svh The testbench assembly

The environment creates the agent and scoreboard. In connect_phase, it connects agent.monitor.observed to scoreboard.observations.analysis_export. That single connection is what routes actual observations into the FIFO.

## counter_test.svh The run controller

phase.raise_objection(this);
seq.start(env.agent.sequencer);
@(negedge env.agent.driver.vif.clk);
phase.drop_objection(this);

The test creates the environment, starts the sequence, and holds an objection while the test is active. The final falling-edge wait gives the monitor and scoreboard time to process the last rising-edge result before the objection drops.

# Part 6 UVM machinery explained without the mystery

## Factory versus configuration database

type_id::create uses the UVM factory to create a class object or component. Factory registration macros make a type available to that mechanism and permit type overrides in larger environments. uvm_config_db::set/get passes configuration data, here the interface handle. The factory answers “which object should exist?” The configuration database answers “what should this object know or use?”

## The main UVM phases used here

| Phase | What this project does |
| --- | --- |
| build_phase | Creates components, creates the analysis FIFO, and retrieves virtual interfaces. |
| connect_phase | Connects the sequencer to the driver and monitor analysis port to FIFO. |
| run_phase | Starts the sequence and performs clock-time driving, monitoring, and checking. |
| check_phase | Checks that enough observations and required scenarios were completed. |
| report_phase | Prints coverage and scoreboard summary. |

Compilation and elaboration are Verilator operations before simulation, not UVM phases. UVM also defines runtime subphases such as reset, configure, main, and shutdown. This small project keeps its timed behavior in run_phase.

## Objections and why the test does not end too soon

The test raises an objection before starting the sequence. UVM keeps run_phase active while the objection exists. After the sequence completes and the last observation has time to arrive, the test drops the objection. UVM then moves toward check_phase and report_phase. The driver, monitor, and scoreboard loops can be forever loops because UVM stops their run_phase processes when the run phase ends.

## Analysis port and FIFO

The monitor calls observed.write(sample) on a uvm_analysis_port. The environment connects that port to the analysis_export on the scoreboard's uvm_tlm_analysis_fifo. The FIFO preserves observation order. The scoreboard blocks on observations.get(sample) until an item arrives. This is transaction-level communication, not a direct wire connection.

# Part 7 Coverage, results, and limitations

## Checking correctness versus measuring exercise

The scoreboard decides whether observed behavior is correct. The covergroup measures which observed reset and count values occurred. A test can be correct for every observed cycle but still miss an important scenario, so both are useful. The scoreboard also has explicit counters for scenarios such as rollover and resumed counting.

The monitor covergroup contains a reset coverpoint, 16 count-value bins, and a reset-by-count cross. The cross includes combinations that a correct synchronous-reset counter cannot produce after a reset edge, such as reset high with count 7. Therefore its 70.0 percent instance coverage is not a failure or a target to maximize blindly. All 16 output values were observed.

## Verified result from this project

| Measure | Observed result |
| --- | --- |
| Verilator | 5.052 on macOS Apple Silicon |
| UVM | Accellera IEEE 1800.2-2017-1.0 source, UVM_NO_DPI |
| Build and elaboration | Completed successfully |
| Simulation | Completed at 310 ns, exit code 0 |
| Scoreboard | 31 checks, 0 failures |
| Scenarios | 5 reset edges, 26 increments, 1 rollover, 2 resumptions |
| Count values | All 0 through 15 observed; values_seen = ffff |
| Covergroup | 70.0 percent instance coverage |
| UVM report | 0 errors, 0 fatals |

The log also contains warnings associated with this local UVM and Verilator setup, including component-name checks under UVM_NO_DPI and a stack-size request. They did not cause a functional test error. The 2017 UVM release documents deviations from the standard in its own DEVIATIONS.md.

## What this example does not yet teach

The stimulus is directed. The item field is declared rand, but this sequence does not call randomize().

The monitor and observation use two-state bit fields. This setup is aimed at functional count behavior after reset, not four-state X and Z diagnosis.

The test checks values after rising edges. It does not separately assert that count remains unchanged between edges.

There is one active agent and one sequence. More complex UVM environments add multiple agents, sequences, configuration objects, and reuse patterns.

# Part 8 Reproduce and explore the run

Open a terminal in the project directory. The build script compiles the project copy at rtl/counter.v. It looks for the Accellera UVM src directory in a sibling folder by default; if your UVM sources are elsewhere, set UVM_SRC to the directory containing uvm_pkg.sv before building.

./build.sh
./run.sh

build.sh invokes Verilator with timing and coverage enabled, compiles the UVM package before the project package and top, and stores generated files in build/obj_dir. run.sh executes the binary from build/ and writes build/run.log. The build output is in build/compile.log. You can search run.log for [SUMMARY], [COVERAGE], UVM_ERROR, and UVM_FATAL.

## Small experiments after you understand the baseline

1. Predict the output sequence for reset values 1, 0, 0, 1, 0 by hand. Then compare your table with the scoreboard algorithm.

2. Change one requested reset value in a temporary copy of the sequence and predict how the reset and increment counts will change. Rebuild and observe the log.

3. In a temporary copy of rtl/counter.v, deliberately change + 1 to + 2. Rebuild and watch the scoreboard report the first mismatch. Restore the original RTL afterward.

4. Trace one observation by locating observed.write in the monitor, the connection in the environment, and observations.get in the scoreboard.

# Part 9 A learning route that avoids overload

Do not try to memorize every UVM class or macro at once. Use four passes.

1. Pass 1: Know the hardware. Explain synchronous reset, nonblocking assignment, and rollover without looking at UVM.

2. Pass 2: Follow one item. Read sequence item, sequence, driver, and interface. Translate req.reset and vif.reset into “request value” and “real wire.”

3. Pass 3: Follow one observation. Read monitor, observation, environment connection, FIFO get, and scoreboard comparison.

4. Pass 4: Read the scaffolding. Study factory creation, config_db, agent and environment ownership, phases, and objections.

At the end of each pass, explain it aloud in your own words. If you can describe a full cycle from requested reset to scoreboard result without class names, the class names will become much easier to remember.

# Part 10 Podcast preparation guide

This section is a recording outline, not a prewritten claim that every listener already knows UVM. The intended speaker is a learner who knows basic Verilog and is using one real counter project to understand SystemVerilog and UVM step by step. Keep the examples concrete and pronounce each code name before explaining it.

| Segment | Story to tell | Code anchor |
| --- | --- | --- |
| 1 The counter | What synchronous reset, nonblocking assignment, and 4-bit rollover mean. | rtl/counter.v |
| 2 Why a testbench needs roles | Request, drive, observe, and check are different jobs. | Architecture diagram in Part 3 |
| 3 One instruction card | A sequence item holds reset; a sequence chooses the pattern. | counter_seq_item.svh, counter_sequence.svh |
| 4 From request to wire | Sequencer handshake, req.reset versus vif.reset, falling-edge drive. | counter_driver.svh |
| 5 From wire to evidence | Monitor waits after the rising edge and records real pins. | counter_monitor.svh |
| 6 The independent answer key | FIFO and scoreboard model; show a pass and a deliberate failure. | counter_env.svh, counter_scoreboard.svh |
| 7 The UVM scaffolding | Agent, environment, test, factory, config_db, phases, objections. | counter_agent.svh, counter_test.svh |
| 8 What the result means | 31 checks, rollover, all count values, and why 70 percent cross coverage is not a failure. | build/run.log |

## A concise opening you can adapt

“I know the basics of Verilog, but SystemVerilog and UVM initially looked like too many classes. This project uses a four-bit counter to make the flow visible. A sequence asks for a reset value. A driver puts it on the real wire. A monitor records the counter output. A scoreboard calculates the answer independently. We will follow one clock cycle before we learn the surrounding UVM structure.”

## Questions that make good discussion breaks

If reset changes between edges, does the counter change immediately? Why?

What is the difference between req.reset and vif.reset?

Why does the monitor look at the DUT pins instead of copying the sequence item?

If the scoreboard used the DUT output as its previous expected value, what kind of bug could it miss?

Why do we need both a scoreboard and coverage?

Why can the covergroup report less than 100 percent even when the directed goals pass?

## Accuracy notes for recording

Say “UVM is a SystemVerilog verification library and methodology,” not “UVM is a new hardware language.”

Say “the reset is synchronous and active high”; do not call it an asynchronous reset.

Say “the monitor samples 1 ps after the positive edge to see the nonblocking update.”

Say “31 observed checks” and “30 directed sequence items plus the initial reset edge.”

Do not equate 70 percent covergroup coverage with 70 percent correctness. The scoreboard had zero failures.

# Glossary for quick reference

| Term | Meaning in this project |
| --- | --- |
| DUT | Design under test: the four-bit counter. |
| Transaction / item | An object holding one requested reset value. |
| Sequence | Code that chooses and sends a series of items. |
| Sequencer | UVM component that coordinates item delivery. |
| Driver | Component that changes reset on the real interface. |
| Monitor | Passive component that reads actual pins. |
| Virtual interface | Class-held reference to the already existing interface instance. |
| Analysis port | One-way UVM publisher used by the monitor. |
| TLM FIFO | Queue between monitor publication and scoreboard consumption. |
| Scoreboard | Independent expected-value model and comparator. |
| Factory | UVM mechanism used by type_id::create to create objects/components. |
| config_db | UVM mechanism used here to pass the interface reference. |
| Objection | Keeps UVM run_phase alive while the test is active. |
| Coverage | Records which observed situations were exercised. |
| NBA | Nonblocking-assignment update region in SystemVerilog simulation. |

Start in the project directory with rtl/counter.v. The most useful first UVM pair is counter_seq_item.svh followed by counter_driver.svh. The goal is to understand the path from request to real wire to observed output to independent check, then return to the surrounding UVM mechanics.